# PROPELLER PAINTING

(REPRODUCTION FROM INFORMATION OBTAINED FROM BLACK DOG PROPELLERS 4/6/2001)

Painting your propeller will degrade the performance of the propellers.  Barnacles on the other hand will degrade the performance more than properly applied paint.  If you use the boat a lot painting is not necessary.  if you have the bottom regularly cleaned then painting is not necessary.  On the other hand if you are like most of us and use the boat not as often as you would like then painting may be helpful.  The steps are as follows:

- The props will be clean when you receive them [from Black Dog Propellers] save for a light coat of oil.  Clean this oil film off the propeller with alcohol or acetone.

- Choose a good quality Zinc Chromate primer and lightly coat the propellers.

- The anti-fouling paint to use on propellers is sold under various trade names as “Outdrive Anti-fouling Paint” in spray cans.  Spray 2-3 light even coats of paint on the prop taking care not to get any paint into the bore of the hub.

- Allow at least 48 hours drying time before putting the propellers into service.

- Regular anti-fouling paint is a poor choice in that it tend to “spin off” the propellers quickly and can not be applied as evenly as spray paint.