| ## H&S Marine Services | # Mailing Address # P.O. Box 573 # Lake Jackson, Texas 77566 Phone: (979) 849-3088 Fax: (979) 849-9243 | Shipping Address 6000 Hwy 288-Bus. South Angleton, Texas 77515 Email: sales@hsmarineservices.com www.hsmarineservices.com |
| --- | --- | --- |

READY TO REPOWER?

BEAT THE HIGH COST OF NEW DIESELS WITH A DODGE CUMMINS CONVERSION!

If you can’t justify the high cost of repowering with new diesels, then it’s time to convert a truck engine to a marine engine, with H&S Marine Services’ marine conversion kit. Savings of 40% to 50% can be realized over the cost of a new diesel marine engine!

Diesel engines have many advantages as marine power plants, including longer life, better fuel economy, less maintenance and higher reliability. Since diesel fuel has a higher flash point than gasoline, it is also a safer fuel to use. Most of the time, diesel fuel also offers considerable savings at the fuel pump.

The Dodge Cummins 6BT series is an inline six cylinder diesel built rugged for serious hard work, and used worldwide in truck, marine and industrial applications. It is a very reliable engine with low RPM and high torque. Even the 160 HP version produces over 400-ft. lbs. of torque. This means your boat can turn a large propeller at slower engine speeds than with gas power for higher cruising speeds, better economy, longer engine life, and more power in heavy seas. In marine application, this engine will weigh approximately 1100 lbs.

Need something different than our standard kit? Our conversion kits can be used in keel cooled, dry exhaust, and other applications. We can supply necessary parts for many different applications. Could you use A raw water pump along with the one for the engine, for bait well etc? Our raw water pump is mounted where the power steering pump mounts on side of engine, and is gear driven. If you need a second pump we can mount on front of engine. Tell us what you need, and we will price out kit to your specs.

Are you tired of paying high prices for marine parts? If any parts were ever needed, belts, hoses and engine parts can be found at your local discount part stores.

Here are some other benefits of converting to our kits:

- Fuel less flammable

- Better fuel economy

- More reliable

- Less maintenance

- Value of boat increases with diesel power

- Possible insurance rate reduction.

WOULD YOU BE INTERESTED IN A COST-EFFICIENT, LIGHT- WEIGHT

MARINE DIESEL?

The Dodge Cummins engines 1989-1997 will range from 160 HP to 215 HP in stock application. The 1989-1993 engines can be brought up to about 200 to 215 HP with adjustments to injector pump and after-cooler installed. If you need more than 215 HP, this can be achieved by reworking the injector pump and replacing injectors. You can also use the 1994-1997 engines with the Big Bosch injector pump, with 160 to 215 HP available from truck depending on year model. The HP can be increased. The 1998-2001 24 valve with 235HP out of truck, with more available.

When deciding what HP you would like to have, remember the more stock you leave the engine, the better economy, performance, and reliability you will receive. This relates to any engine, gas or diesel.  We have kits that will increase the HP on the 1994-1997 engines to 180, 210, 230, 250 +. The 24 Valve engine can also be increased to over 250 HP. At 250 HP, we recommend an exhaust temperature pyrometer gauge kit. Under certain conditions such as, exhaust restriction or too much prop, it is possible the exhaust temperature would rise too high. If this situation should happen, this kit would let you know so the throttle could be changed to drop temperature until problem was corrected (this potential problem relates to any diesel repower application, not just our application).

For customers wanting to use rubber isolator mounts instead of bolting engine solid, don’t use just any rubber mounts. In our accessories we list front and rear engine mounts that were designed to carry the weight, engine torque, and prop thrust of these engines.

There is not any mechanical reason the Dodge 6BT Cummins will not give good results in a marine application.  We have some engines that are going on their third year in charter service with no mechanical problems. Our kits are in boats from 25’ to 35’ and even a 55’ sailboat in Hawaii. It is possible to have 300HP or more, but we hesitate to go over 265-270 HP. That does not mean that if you go to 300HP or more you will have a problem.

COMING SOON

We have an adapter shaft that will connect to the Volvo Stern Drive, and we are working on other stern drive applications.

We can adapt to generators, water pumps, and industrial applications. Let us know what application you need. We will be glad to work with you on your particular application.

________________________________________________________________

1989—2001 KIT CONTENTS

1----Water Cooled Exhaust Manifold

6----Exhaust Manifold Mounting Gaskets

12----Exhaust Manifold Bolts

1----Water Cooled Turbo Exhaust Housing, Gasket, & 4 Bolts

1----Turbo Oil Supply Line

1----Turbo Oil Return hose, fitting, gasket, & Clamps

4---- ¾” Brass Fittings, Hose & Clamps (from head to water cooled turbo)

1----Heat Exchanger  w/ Day tank (5” copper/nickel alloy tubes)

2----Heat Exchanger Metal Mounting Brackets w/ 4—3/8 SS Bolts & Nuts

3----Heat Exchanger Rubber Mounting Assembly, 6- ss bolts w/nuts

2----2 ¼” Rubber Hoses 90/Pipe & Clamps (from exchanger to engine suction)

1----Thermostat Housing

1----Thermostat & seal

1----1 ¾” Hose & Clamps (from thermostat housing to heat exchanger)

1----#3 Marine Transmission Mount Adapter assembly

(our #3 adapter fits the Twin Disc 5011A and 5000 Velvet Drive dimensions) We can probably adapt to your transmission if necessary. In the Volvo application, assembly is machined to fit the Volvo flange housing.

1----Flywheel

2----Front metal Engine Mounts (use existing bolts from truck mounts)

2----Rear metal Engine Mounts & 6 Mounting Bolts

1----Throttle Cable Mounting Bracket & SS cable clamp

2----3/4” Pipe Plugs

1----2 ½” Piping Assembly from Turbo to Engine Intake

1----Water Cooled 4” OD SS Exhaust Elbow, Gasket & 4 Bolts

1----Raw Water Pump Assembly

1----Piping Assembly from Raw Water Pump to Heat Exchanger (hoses/clamps)

2----3/4” Hose fitting, clamps and hose from exhaust manifold to engine water intake

1----oil pan drain hose assembly

Any bolts in kit that require a nut, both will be furnished in Stainless Steel.

We have tried to furnish everything needed to complete this conversion kit.

________________________________________________________________

We can also help you with your transmission needs—price quote with kit purchase

ITEMS NEEDED ON ENGINE

When locating an engine from 1989 to 1997 these items listed below need to be on the engine, as they do not come in the kit:

Run solenoid on injector pump

Turbo

Alternator

Alternator bracket

Alternator belt tensioner (spring loaded tensioner)

Starter (see #B in note section

Flywheel bell housing (see #B in note section)

Water inlet housing

Engine wiring harness and engine computer on 1998 to 2001 engines

Engine Fan Assembly.

(It is better to use the front pulley assembly on truck engine. Just remove fan from assembly)(Engines can come with different diameter crankshaft pulleys, This is why we do not furnish belt in kit) Correct belt length can be purchased at local part store.

ITEMS NOT NEEDED

Truck Exhaust manifold

Truck Transmission cooler

Wiring harness on 1989 to 1997

Alternator belt

Vacuum, Steering pump assembly

NOTE SECTION

#A---A water-cooled exhaust turbo housing comes in kit. The truck turbo is removed from the truck exhaust housing side and installed into water-cooled exhaust housing provided in kit.  With this set up you will have the boost to increase the HP to the 250-260 HP Range. If you have to have more HP than this we can give you credit for water-cooled turbo Housing in kit and you can purchase complete Marine turbo assembly for 300HP or higher to have needed boost pressure for more HP.

#B---Our kit fits the 1994 - 1997 flywheel housing and starter. If customer wants to use the 1989 - 1993 engine, he will need to locate a starter and flywheel housing for the 1994 - 1997 engine to put on the earlier engine. We can assist you in locating these parts

REPOWERING TIPS

Below are a few steps that will help you achieve the best performance, reliability, and satisfaction with a repower application. This relates to any repower application not just this application.

- Use transmission rated for your application.

- Try to keep engine room temperature within about 30 degrees of outside temp. The cooler the air going into engine, the better the performance.

- Prop boat for max RPM with full load.

- Check exhaust temperature and pressure, at rated speed and load. This will let you know if you are propped right and exhaust size is correct.

ENGINE SPECIFICATIONS

- Length----39” from front pulley to face of bell housing

2.  Length----50” from front crankshaft pulley to turbo piping (over all length)

- Width-----1988 to 1993 without turbo after-cooler-----26 ½”

1994 to 1997 without turbo after-cooler-----31 ½”

- Width-----1988 to 1993 with turbo after-cooled----------30 ¾”

1994 to 1997 with turbo after-cooler-----------31 ½”

- Front engine mounts ¾” slot width-----17” min. to 28” max.

- Rear engine mounts ¾” slot width------23” min. to 27” max.

- Center of front mount to center of rear mount----32”

- Center of front crankshaft to top of highest point of engine---21 ½”

- Highest point of engine to bottom of front engine mounts----25”

WARRANTY

The exhaust manifolds have a one-year warranty. The water-cooled turbo housing has a six- month warranty. The Sherwood raw water pump has a six-month warranty (excluding a worn impeller). The heat exchanger, after-cooler, water-cooled SS exhaust elbow, and all brackets carry a one-year warranty.

TERMS

On kit purchases, a 30% deposit is required. Upon receipt of deposit, we will inform you of the anticipated delivery based on parts availability. If the delivery schedule does not meet your time requirements, your deposit will be refunded. Balance is due before shipment or within 15 days of completion of kit ready for shipment.