The STP Test

The results are in for the great STP Test. The controversy was, will 
STP, when applied to your prop, keep barnacles from attaching 
themselves. And the answer is, possibly.

Here's the deal. For all of you with senior memories, that is loss 
of the short term, I agreed to set up a test in some of the most 
barnacle prone proving grounds I know of, and that was in my back 
yard. The location is St. Petersburg, Florida, a canal between Tampa 
Bay and Boca Ciega Bay. Water flow through the twice daily change is 
at times, 1+ knots. The water temperature began at about 80 degrees, 
but as of yesterday is 84. The test ran from Thursday April 25th to 
Sunday May 5th. 

I suspended a piece of 1" x 36" flat stock aluminum in the water at 
about prop depth. On one half, all sides, I coated it with what I 
consider would be a liberal amount of STP. It was not running off, 
but fully coated. The other half, I left bare. This stayed 
suspended in the water for 10 days, at prop depth, subject to the 
same lighting and current conditions as my boat. 

Upon retrieval, the bare portion was covered with small barnacles and 
other encrustations the size of 80 grit sandpaper. I would say 100% 
covered. These were firmly attached, yet could be removed with a 3M 
scrunge pad, much like a diver would use on your bottom. The STP 
coated portion was another matter. It was in blotches, for lack of a 
better term, about 40% covered. The STP seemed to be washed off in 
some sections, yet was still slick. The barnacles had no preference 
to the washed off area over the fully coated area. The growth could 
be wiped off with your finger as opposed to the 3M pad required on 
the bare portion.

This leaves me with some opinions and conclusions. The STP seems to 
help, although on the washed off portions, I wonder how long it would 
take a spinning prop to polish off the STP. Having said that, on the 
other hand, anything clinging to the STP would most certainly be 
slung off with prop rotation as it was so easily wiped off the test 
strip with my finger alone. 

I have a diver on the bottom every 3 to 4 weeks in the summer and 4 
to 6 weeks in the winter, those seasons are dictated by water 
temperature. This might work as a renewable coating that can be 
placed on under water at each cleaning. I will have a discussion 
with the dive company to see if they are willing to try this. They 
would have to get over the idea that they are putting oil in the 
water as that is not really true in terms of sheen. There is no 
multi-colored rainbow type slick when you put STP in the water. A 
diver could use a mitt of some sort to apply it.

There you have it.